- Anthem I225 remote codes and RS232 control

# IR commands

Remote transmitter IC – Princeton Technology PT2222

Remote Custom Code – 6666 (Hex)

| Key name | Remote Input pin | Action type | Action position | Action time |
| --- | --- | --- | --- | --- |
| All keys | U11 – P43/Pin12 | PT2222 Code | Forward | < 0.1 sec |

| Key name | Code | Performance |
| --- | --- | --- |
| On ( Power ) | 05 | Single Key |
| Off ( Power ) | 01 | Single Key |
| Volume UP | 19 | Single Key |
| Volume DN | 1F | Single Key |
| Input 1 | 55 | One of Input |
| Input 2 | 56 | One of Input |
| Input 3 | 57 | One of Input |
| Input 4 | 58 | One of Input |
| Input 5 | 59 | One of Input |
| Tape | 5A | One of Input |
| Balance | 5C | One of Input |
| Stereo | 5D | One of Input |
| Reserve | 5E | Cycle Key |
| Mute | 5B | Cycle Key |
| Defeat | 5F | Cycle Key |

RS-232 commands

## Power ON/OFF

Power Status Read Command – RP (Read Power)

| Command | Write | Auto Return Read |
| --- | --- | --- |
| PC Power ON | W 1 1 | PC 1 1 |
| Key Power ON |  | K 1 1 |
| Remote Power ON |  | R 1 1 |
| Trigger Power ON |  | T 1 1 |
| PC Power OFF | W 1 0 | P 1 0 |
| Key Power OFF |  | K 1 0 |
| Remote Power OFF |  | R 1 0 |
| Trigger Power OFF |  | T 1 0 |
| PC Power Toggle --- ON | W 1 10 | PC 1 1 |
| PC Power Toggle --- OFF | W 1 10 | PC 1 0 |

Input Key Functions

Input Function Status Read Command – RI (Read Input)

| Condition | Write | Auto Return Read |
| --- | --- | --- |
| PC Input 1 | W 2 1 | PC 2 1 |
| Key Input 1 |  | K 2 1 |
| Remote Input 1 |  | R 2 1 |
| PC Input 2 | W 2 2 | PC 2 2 |
| Key Input 2 |  | K 2 2 |
| Remote Input 2 |  | R 2 2 |
| PC Input 3 | W 2 3 | PC 2 3 |
| Key Input 3 |  | K 2 3 |
| Remote Input 3 |  | R 2 3 |
| PC Input 4 | W 2 4 | PC 2 4 |
| Key Input 4 |  | K 2 4 |
| Remote Input 4 |  | R 2 4 |
| PC Input 5 | W 2 5 | PC 2 5 |
| Key Input 5 |  | K 2 5 |
| Remote Input 5 |  | R 2 5 |
| PC Tape Input | W 2 6 | PC 2 6 |
| Key Tape Input |  | K 2 6 |
| Remote Tape Input |  | R 2 6 |
| PC Balance Input | W 2 7 | PC 2 7 |
| Key Balance Input |  | K 2 7 |
| Remote Balance Input |  | R 2 7 |
| PC Stereo Input | W 2 8 | PC 2 8 |
| Key Stereo Input |  | K 2 8 |
| Remote Stereo Input |  | R 2 8 |
| PC Reserve SW ON | W 5 1 | PC 5 1 |
| Key Reserve SW ON |  | K 5 1 |
| Remote Reserve SW ON |  | R 5 1 |
| PC Reserve SW OFF | W 5 0 | PC 5 0 |
| Key Reserve SW OFF |  | K 5 0 |
| Remote Reserve SW OFF |  | R 5 0 |
| PC Reserve SW Toggle --- ON | W 5 1 | PC 5 1 |
| PC Reserve SW Toggle --- OFF | W 5 0 | PC 5 0 |
| PC Defeat ON | W 6 1 | PC 61 |
| Key Defeat ON |  | K 6 1 |
| Remote Defeat ON |  | R 6 1 |
| PC Defeat OFF | W 6 0 | PC 6 0 |
| Key Defeat OFF |  | K 60 |
| Remote Defeat OFF |  | R 6 0 |
| PC Defeat Toggle --- ON | W 6 10 | PC 6 1 |
| PC Defeat Toggle --- OFF | W 6 10 | PC 6 0 |

## Volume UP/DOWN

(Volume UP/DOWN Status Read Command – No use)

| Condition | Write | Auto Return Read |
| --- | --- | --- |
| PC Volume UP | W 4 1 | PC 4 1 |
| Remote Volume UP |  | R 4 1 |
| PC Volume DOWN | W 4 0 | PC 4 0 |
| Remote Volume DOWN |  | R 4 0 |

Note: PC Volume UP or DOWN command is a 0.3 – 0.5 sec pulse.

## Mute ON/OFF

Mute Status Read Command – RM (Read Mute)

| Condition | Write | Auto Return Read |
| --- | --- | --- |
| PC Mute ON | W 3 1 | PC 3 1 |
| Remote Mute ON |  | R 3 1 |
| PC Mute OFF | W 3 0 | PC 3 0 |
| Remote Mute OFF |  | R 3 0 |
| PC Mute Toggle --- ON | W 3 10 | PC 3 1 |
| PC Mute Toggle --- OFF | W 3 10 | PC 3 0 |

Overload Condition

Overload Status Read Command – ROL (Read Overload)

| Condition | Write | Auto Return Read |
| --- | --- | --- |
| L Overload ON |  | L 7 1 |
| L Overload OFF |  | L 7 0 |
| R Overload ON |  | R 7 1 |
| R Overload OFF |  | R 7 0 |

Overheat Condition

Overheat Status Read Command – ROH (Read Overheat)

| Condition | Write | Auto Return Read |
| --- | --- | --- |
| L Overheat ON |  | L 8 1 |
| L Overheat OFF |  | L 8 0 |
| R Overheat ON |  | R 8 1 |
| R Overheat OFF |  | R 8 0 |

## RS-232 communication read for the status when Power ON

and All Read command

All Read Command --- RALL

The Auto Return Read Sequence As Below :

(1) Power ON --- PC 1 1 or K 1 1 or R 1 1 or T 1 1

(2) Input --- PC 2 1 ,K 2 1,R 2 1 , PC 2 2 ,K 2 2, R 2 2 ,PC 2 3,K 2 3,

- R 2 3,PC 2 4,K 2 4,R 2 4,PC 2 5,K 2 5,R 2 5,PC 2 6,K 2 6,

R 2 6,PC 2 7,K 2 7,R 2 7,PC 2 8,K 2 8,R 2 8 ,

(3) Mute --- PC 3 0,R 3 0,PC 3 1,R 3 1 ,

(4) Defeat --- PC 6 0,K 6 0,R 6 0,PC 6 1,K 6 1,R 6 1,

(5) Reserve SW --- PC 5 0,K 5 0,R 5 0, PC 5 1,K 5 1,R 5 1 ,

(6) L Overload --- L 7 0, L 7 1 ,

(7) R Overload --- R 7 0, R 7 1 ,

(8) L Overheat --- L 8 0 , L 8 1 ,

(9) R Overheat --- R 8 0 , R 8 1 ,

(10) < CR >

Read All Command “RALL” when Power OFF

Auto Return Read Sequence:

- PC10 or K10 or R10 or T10

- < CR >

## First Power ON Default Setting

( 1 ) PC11 or K11 or R11 or T11

( 2 ) K21

( 3 ) K30

( 4 ) R30

( 5 ) K50

( 6 ) L70

( 7 ) R70

( 8 ) L80

( 9 ) R80

( 10 ) < CR >